MA STEK

Power Anywhere & Anytime

VENUS-E

# AC Coupled ESS

- 2.5kW Multi On-grid, Plug & Play

- 5.12kWh Battery Capacity

- 2.5kW Back-up & Off-grid

- Anti Feed-in ,100% Self-consumption (By CT)

- RS485 EMS, Peak Shaving and Valley Filling

- IP65 Waterproof

Grid

*

Meter

CT

...

Anti Feed-in

Smart Backup Box (Optional)

UPS: 20ms automatically switching while grid off.

100 % Self-consumption Guaranteed

Marstek Energy Co., Limited

Add: BLDG#A, Changfang Industrial Park, No.2 Guihua 5th Road, Pingshan District, Shenzhen, Guangdong, China Tel: 86-755-28398778	Email: market@marstekenergy.com	Web: www.marstekenergy.com

SPECIFICATIONS

_J_

600mm

E

.E,.

N

r-

| Model | VENUS-E |
| --- | --- |
| Rated Voltage | 51.2V(l6S) |
| Capacity | 5120Wh |
| Life Cycle (Times) | >6000 (25'C) |
| Battery Type | LiFePO, |
| DoD | 90% |
| Current | 60A |
| Rated Input/ Output Power | 2.5kW |
| Operating Phase | L / N / PE |
| Rated Grid Voltage | 220V / 230V |
| Grid Voltage Range | 187V~253V |
| Rated Grid Frequency | 50Hz / 60Hz |
| Rated Grid Input/Output Current | 11.4A / 10.9A |
| Max. Input/Output Current | 13.4A |
| Power Factor | > 0.99 (Default)/ 0.8 leading~0.8 lagging (Adjustable) |
| THDi | <3% |
| Rated Off-grid Output Power | 2.5kW |
| Max. Apparent Output Power | 2.5kVA |
| Max. Output Current | 11.3A |
| Rated Output Voltage | 220V |
| Rated Output Voltage Freq. | 50Hz/60Hz |
| THDu (Linear Load) | <3% |
| Battery-AC Side Max. Efficiency | >93.5% |
| Protection Levei |  |
| Overvoltage/Voltage Withstand Levei | DC li/ AC III |
| lsolation Type | Non-lsolated |
| Operating Temperature Range | -20'C~ +60'C |
| Ralative Humidity | 0-95% |
| lngress Protection | IP65 |
| Cooling Strategy | Natural Convection |
| Max. Operating Altitude | 2000m |
| On-grid Connection Standard | VDE 4105/0124,TOR R25, NF EN50549-10,Cl0/110,UNE217001/217002,NTS 631,CEI0-21,PETREE,PN-EN 50549-1/-2 |
| Regulatory/EMC | IEC/EN 62109-1/-2,IEC/EN 61000-6-1/-2-/3-/4,IEC/EN 61000-3-2/-3,EN 300 328,EN 301489-1/-17, EN 50665,EN 62920,CB 62619, EN 50549-1 |
| Dimension | 600*742*227mm |
| Weight | 60±0.Skg |
| On-grid AC Connection | Three-wire Household Plug (Eurol6A) |
| Display | LED |
| Supported Communication lnterface | WIFI & RS485 (Waterproof aviation plug) |